# Orange Pi 5 Plus Installation and usage Resources

# OS Installation

## Step 1 : Downloading OS image :

Download the image from this link : https://joshua-riek.github.io/ubuntu-rockchip-download/boards/orangepi-5-plus.html

Step 2 : Flashing the image :

- Download Balena etcher https://etcher.balena.io/#download-etcher

- Insert SD card to SD card reader and plug it to your PC.

- Open Balena Etcher, select downloaded image , select target sd card and click on flash.

# ROS2 Installation :

- Set Locale UTF file type

locale  # check for UTF-8

sudo apt update && sudo apt install locales

sudo locale-gen en_US en_US.UTF-8

sudo update-locale LC_ALL=en_US.UTF-8 LANG=en_US.UTF-8

export LANG=en_US.UTF-8

locale  # verify settings

- Add Remote Repository

sudo apt install software-properties-common

sudo add-apt-repository universe

sudo apt update && sudo apt install curl -y

export ROS_APT_SOURCE_VERSION=$(curl -s https://api.github.com/repos/ros-infrastructure/ros-apt-source/releases/latest | grep -F "tag_name" | awk -F\" '{print $4}')

curl -L -o /tmp/ros2-apt-source.deb "https://github.com/ros-infrastructure/ros-apt-source/releases/download/${ROS_APT_SOURCE_VERSION}/ros2-apt-source_${ROS_APT_SOURCE_VERSION}.$(. /etc/os-release && echo $VERSION_CODENAME)_all.deb" # If using Ubuntu derivates use $UBUNTU_CODENAME

sudo apt install /tmp/ros2-apt-source.deb

- Install  ROS2

sudo apt update

sudo apt upgrade

sudo apt install ros-humble-desktop

- Add path to sytem

echo "source /opt/ros/humble/setup.bash" >> ~/.bashrc

source ~/.bashrc

Test Example nodes

- Run demo Publisher /talker

ros2 run demo_nodes_cpp talker

- Rune demo Subscriber/listener

ros2 run demo_nodes_py listener

- Create a new workspace

mkdir -p ~/ros2_ws/src

cd ~/ros2_ws

# Install Robomaster ROS2 Package

- Install Dependencies

sudo apt install \

ros-humble-xacro \

ros-humble-launch-xml \

ros-humble-cv-bridge \

ros-humble-launch-testing-ament-cmake \

ros-humble-robot-state-publisher \

ros-humble-joint-state-publisher \

ros-humble-joint-state-publisher-gui \

ros-humble-joy \

ros-humble-joy-teleop \

ros-humble-joy-linux

- Install Robomaster SDK Python Drivers

sudo apt install libopus-dev python3-pip

python3 -m pip install -U numpy numpy-quaternion pyyaml

python3 -m pip install git+https://github.com/jeguzzi/RoboMaster-SDK.git

python3 -m pip install git+https://github.com/jeguzzi/RoboMaster-SDK.git#"egg=libmedia_codec&subdirectory=lib/libmedia_codec"

- Install ROS2 Package for robomaster.

cd ~/ros2_ws/src

git clone https://github.com/jeguzzi/robomaster_ros.git

cd ~/ros2_ws/

source /opt/ros/humble/setup.bash

colcon build

- Connect PC/jetson wifi to robomaster wifi

- Launch Robomaster Node

source ./install/setup.bash

ros2 launch robomaster_ros main.launch model:=ep conn_type:=ap

For USB cable, use conn_type:NDIS

# Install RP lidar ROS2 package and Drivers

- Install Package

cd ~/ros2_ws/src

git clone https://github.com/Slamtec/rplidar_ros.git

cd ~/ros2_ws/

source /opt/ros/humble/setup.bash

colcon build --symlink-install

echo "source ros2_ws/install/setup.bash" >> ~/.bashrc

source ~/.bashrc

- Launch the node

sudo chmod 777 /dev/ttyUSB0

cd src/rpldiar_ros/

source scripts/create_udev_rules.sh

ros2 launch rplidar_ros view_rplidar_a2m12_launch.py

# Install realsense drivers and package

- Install librealsense2 (realsense drivers)

sudo apt-get install libssl-dev libusb-1.0-0-dev libudev-dev pkg-config libgtk-3-dev

sudo apt-get install libglfw3-dev libgl1-mesa-dev libglu1-mesa-dev at

sudo apt-get install git wget cmake build-essential

git clone https://github.com/IntelRealSense/librealsense.git

cd librealsense

sudo cp config/99-realsense-libusb.rules /etc/udev/rules.d/

sudo udevadm control --reload-rules && sudo udevadm trigger

mkdir build && cd build

cmake ../ -DFORCE_RSUSB_BACKEND=true -DCMAKE_BUILD_TYPE=release -DBUILD_EXAMPLES=true -DBUILD_GRAPHICAL_EXAMPLES=true

sudo make uninstall && make clean && make -j5 && sudo make install

Note This will take some time (1-2 hours)

- Install ROS2 Package

mkdir -p ~/realsense_ws/src

git clone https://github.com/IntelRealSense/realsense-ros.git -b ros2-master

cd ~/realsense_ws

sudo apt-get install python3-rosdep -y

sudo rosdep init # "sudo rosdep init --include-eol-distros" for Foxy and earlier

rosdep update # "sudo rosdep update --include-eol-distros" for Foxy and earlier

rosdep install -i --from-path src --rosdistro $ROS_DISTRO --skip-keys=librealsense2 -y

colcon build

- Lunch Node :

source install/setup.bash

ros2 launch realsense2_camera rs_align_depth_launch.py

# Deploying Pytorch Model on Orange Pi NPU

There is 2 Stage Workflow to deploy model on orange pi . First you convert your model from pytorch to rknn format for orange pi NPU on your Intel x86 PC/laptop

- Stage 1 PC/laptop  Setup (Model Conversion)

Complete Flow is Pytorch -> ONNX -> RKNN

- Convert to ONNX format

python3 -m pip install onnx

- Convert Model from Pytorch to onnx

import torch

import torchvision

# Load the ResNet18 model from torchvision

model = torchvision.models.resnet18(pretrained=True)

# Set the model to evaluation mode

model.eval()

# Define the input shape (batch size = 1, channels = 3, height = 224, width = 224)

input_shape = (1, 3, 224, 224)

# Create a dummy input tensor

dummy_input = torch.randn(*input_shape)

# Export the model to ONNX format

torch.onnx.export(model,

dummy_input,

"resnet18.onnx",

export_params=True,

opset_version=11,  # ONNX opset version

input_names = ['input'],   # input name

output_names = ['output'])  # dynamic batch size for output

print("ResNet18 model exported to resnet18.onnx")

- Install RKNN toolkit to convert onnx model to .rknn format

python3 -m pip install onnx

sudo apt-get install libxslt1-dev zlib1g-dev libglib2.0 libsm6 libgl1-mesa-glx libprotobuf-dev gcc

git clone --depth=1 https://github.com/airockchip/rknn-toolkit2

cd rknn-toolkit2

PYTHON_VERSION=$(python3 -c 'import sys; print(f"cp{sys.version_info.major}{sys.version_info.minor}")')

python3 -m pip install -r rknn-toolkit2/packages/x86_64/requirements_${PYTHON_VERSION}-2.3.2.txt

PYTHON_VERSION=$(python3 -c 'import sys; print(f"cp{sys.version_info.major}{sys.version_info.minor}-cp{sys.version_info.major}{sys.version_info.minor}")')

python3 -m pip install rknn-toolkit2/packages/x86_64/rknn_toolkit2-2.3.2-${PYTHON_VERSION}-manylinux_2_17_x86_64.manylinux2014_x86_64.whl

- Convert to rknn

  - Prepare dataset.txt : Create text Document with 1 or 2 images paths for conversion (These should be the images the model was trained for

/path/to/image1.jpg

/path/to/image2.jpg

...

- Convert Model

For float16

import numpy as np

from rknn.api import RKNN

import uuid

ONNX_MODEL = 'resnet18.onnx'

RKNN_MODEL = 'resnet18.rknn'

INPUT_SHAPE = [1, 3, 224, 224]

TARGET_PLATFORM = 'rk3588'

rknn = RKNN()

# Confihgure input tensor normalization , these should be configured deending upon your dataset pre-processing

rknn.config(

mean_values=[[0, 0, 0]],  # Default mean values for normalization

std_values=[[255, 255, 255]],  # Default std values for normalization

target_platform=target_platform

)

ret = rknn.load_onnx(model=ONNX_MODEL, input_size_list=[INPUT_SHAPE])

ret = rknn.build(do_quantization=False)

ret = rknn.export_rknn(f'{RKNN_MODEL}_float.rknn')

- For Quantization int8

For quantization we will use dataset.txt

import numpy as np

from rknn.api import RKNN

import uuid

ONNX_MODEL = 'resnet18.onnx'

RKNN_MODEL = 'resnet18.rknn'

INPUT_SHAPE = [1, 3, 224, 224]

DATASET_PATH = './dataset.txt'  # Path to dataset file for quantization (optional)

TARGET_PLATFORM = 'rk3588'

rknn = RKNN()

# Confihgure input tensor normalization , these should be configured deending upon your dataset pre-processing

rknn.config(

mean_values=[[0, 0, 0]],  # Default mean values for normalization

std_values=[[255, 255, 255]],  # Default std values for normalization

target_platform=target_platform

)

ret = rknn.load_onnx(model=ONNX_MODEL, input_size_list=[INPUT_SHAPE])

ret = rknn.build(do_quantization=True, dataset=DATASET_PATH)

ret = rknn.export_rknn(f'{RKNN_MODEL}_int8.rknn')

- Step 2 :  Mode Deployment

- Install RKNN toolkit on Orange Pi

git clone --depth=1 https://github.com/Pelochus/ezrknn-toolkit2

cd ezrknn-toolkit2/

pip3 install opencv-python

pip3 install -r rknn-toolkit2/packages/arm64/arm64_requirements_cp310.txt

pip install rknn-toolkit-lite2/packages/rknn_toolkit_lite2-2.3.0-cp310-cp310-manylinux_2_17_aarch64.manylinux2014_aarch64.whl

pip install rknn-toolkit2/packages/arm64/rknn_toolkit2-2.3.0-cp310-cp310-manylinux_2_17_aarch64.manylinux2014_aarch64.whl

- Run inference

Demo Inference example

import numpy as np

from rknn.api import RKNN

import cv2

import time

model_path = '/home/orangepi/Downloads/resnet18_fp16.rknn'

image_path = '/home/orangepi/Downloads/n01491361_tiger_shark.JPEG'

target_platform = 'rk3588'

class_labels = [f"class_{i}" for i in range(1000)]

rknn = RKNN()

print('--> Configuring model')

if rknn.config(target_platform=target_platform) != 0:

print('Failed to configure RKNN model')

exit(1)

print('Configuration done')

print('--> Loading RKNN model')

if rknn.load_rknn(model_path) != 0:

print(f'Failed to load RKNN model: {model_path}')

exit(1)

print('Model loaded successfully')

print('--> Initializing runtime environment')

if rknn.init_runtime(target_platform) != 0:

print('Failed to initialize runtime environment')

exit(1)

print('Runtime environment initialized')

img = cv2.imread(image_path)

if img is None:

print(f'Failed to load image: {image_path}')

exit(1)

img = cv2.cvtColor(img, cv2.COLOR_BGR2RGB)

img = cv2.resize(img, (224, 224))

img = np.expand_dims(img, axis=0)

print('--> Running inference')

t1 = time.time()

outputs = rknn.inference(inputs=[img])

t2 = time.time()

print(f"Executed in {(t2-t1)*1000:.2f} ms")

if outputs is not None:

outputs = [np.array(out) for out in outputs]

print('Inference completed successfully')

for i, output in enumerate(outputs):

print(f'Output {i} shape: {output.shape}')

output = outputs[0].flatten()

pred_class_idx = np.argmax(output)

confidence = output[pred_class_idx]

class_name = class_labels[pred_class_idx]

print(f'Predicted class: {class_name}, Confidence: {confidence:.4f}')

else:

print('Inference failed')

rknn.release()

print('RKNN resources released')